Low-Level Algorithmic Logic: WhatsApp Metered Data Integration for Non-Metered Sites

Purpose

Define production-ready low-level logic to use WhatsApp-received metered data in scheduler flow for non-metered sites (primarily global2 style sites).

1. Global Variables (Default Values)

ENGINE_INTERVAL_MIN = 15

TZ = Asia/Kolkata

WHATSAPP_TABLE_NAME = plant_whatsapp_meter

WHATSAPP_MAX_STALENESS_MIN = 45

WHATSAPP_MAX_FUTURE_SEC = 120

WHATSAPP_TIME_SNAP_TOL_SEC = 120

WHATSAPP_MIN_MW = -0.20

WHATSAPP_MAX_MW_FACTOR = 1.20

REGEN_MIN_DEVIATION_MW = 0.20

REGEN_COOLDOWN_BLOCKS = 1

WEIGHT_INTRADAY_BASE = 0.92

WEIGHT_WHATSAPP_BASE = 0.08

WEIGHT_WHATSAPP_MAX = 0.35

WHATSAPP_CORRECTION_HORIZON_BLOCKS = 4

IRR_ZERO_TRUST = 0.10

IRR_FULL_TRUST = 0.40

RECENCY_TAU_MIN = 30.0

FORECAST_DYNAMIC_START_FRAC = 0.01

WHATSAPP_DYNAMIC_START_MW = 0.10

2. Input Contracts

WhatsApp payload fields: site_id, actual_mw, event_time, message_id, sender_id.

Scheduler inputs per iteration: latest intraday forecast, weather snapshot + short forecast, control state from DynamoDB, existing schedule state.

3. Message Validation and Normalization

Step 3.1: Reject if site_id mismatch.

Step 3.2: Reject duplicate message_id already processed.

Step 3.3: Reject stale message (age > WHATSAPP_MAX_STALENESS_MIN).

Step 3.4: Reject future event_time (delta > WHATSAPP_MAX_FUTURE_SEC).

Step 3.5: Reject actual_mw outside [WHATSAPP_MIN_MW, plant_capacity_mw * WHATSAPP_MAX_MW_FACTOR].

Step 3.6: Map event_time to 15-min block using tolerance WHATSAPP_TIME_SNAP_TOL_SEC.

4. Dynamic Start Detection (Non-Metered Flow)

Step 4.1: Compute forecast candidate block: first block where intraday_scaled_mw > FORECAST_DYNAMIC_START_FRAC * operational_capacity_mw.

Step 4.2: If valid WhatsApp message exists and actual_mw >= WHATSAPP_DYNAMIC_START_MW, allow immediate dynamic start in current engine block.

Step 4.3: candidate_block = min(forecast_candidate_block, whatsapp_override_block).

Step 4.4: If engine_block >= candidate_block and schedule not exists => create schedule (source=dynamic_start).

5. Reschedule Trigger Using WhatsApp Meter

Step 5.1: For message block b_ref, read current generated schedule value S_ref.

Step 5.2: Compute penalty band MW: band_mw = plant_capacity_mw * penalty_band_fraction.

Step 5.3: Compute allowed range from actual: [actual_mw - band_mw, actual_mw + band_mw].

Step 5.4: If S_ref in range => no reschedule (reason=whatsapp_ignored_in_band).

Step 5.5: If outside range and abs(S_ref - actual_mw) >= REGEN_MIN_DEVIATION_MW and cooldown passed => reschedule (source=whatsapp_out_of_band_adjustment).

6. Base Forecast Calculation with Adaptive Blending

For each target block b:

6.1 intraday = intraday_by_block[b]

6.2 if no valid WhatsApp message OR b beyond correction horizon => base_forecast = intraday

6.3 else compute irradiance trust factor:

irr_factor = clamp((irr_ratio - IRR_ZERO_TRUST)/(IRR_FULL_TRUST - IRR_ZERO_TRUST), 0, 1)

6.4 compute recency factor:

recency_factor = exp(-message_age_min / RECENCY_TAU_MIN)

6.5 compute meter weight:

w_m = clamp(WEIGHT_WHATSAPP_BASE * irr_factor * recency_factor * confidence, 0, WEIGHT_WHATSAPP_MAX)

6.6 w_i = 1 - w_m

6.7 base_forecast = w_i * intraday + w_m * actual_mw

7. Final Schedule Generation

Step 7.1: Apply existing weather-based condition logic (Condition-3) on base_forecast.

Step 7.2: Apply plant control override in order: shutdown > curtailment cap > normal.

Step 7.3: Apply ramp/smoothing constraints and acceptance checks.

Step 7.4: Write schedule CSV and meta JSON.

8. Reason Mapping Rules

dynamic_start + curtailment => curtailment_dynamic_start

abrupt_weather + curtailment => curtailment_abrupt_weather_change

plant_status_change to NORMAL => plant_status_normal

whatsapp in-band => whatsapp_ignored_in_band

whatsapp out-of-band regen => whatsapp_out_of_band_adjustment

9. State Persistence

Persist: last_whatsapp_msg_id, last_whatsapp_event_ts, last_whatsapp_mw, last_whatsapp_block, last_whatsapp_regen_block, dynamic_start_block, plant_status, curtailment_capacity.

10. Fail-Safe Behavior

If WhatsApp message invalid/stale/missing: fallback to pure forecast-driven flow.

No scheduler stop/fail due to WhatsApp parsing failure.

11. Deployment Notes

This logic should be feature-flagged per site (enable_whatsapp_meter=true) and initially enabled only for target non-metered plants.